# EXPERIMENT 4: NS3 - TCP/UDP Communication (Single Client Chatting)

Experiment No: 4

Date: 10-04-2026

## Objectives of the Experiment:

- To understand client-server communication using UDP protocol in NS3

- To create a point-to-point network connection between two nodes

- To implement a UDP client that sends multiple packets to a server

- To implement a UDP server that receives and logs packets

- To analyze packet transmission timing and sequence

- To document the communication process with screenshots

## Introduction:

UDP (User Datagram Protocol) is a connectionless transport layer protocol. Unlike TCP, UDP does not guarantee reliable delivery but offers lower latency and overhead. This experiment demonstrates a simple client-server chat using UDP where a client sends multiple packets to a server, and the server acknowledges receipt by printing packet information.

### UDP Characteristics:

| Feature | Description |
| --- | --- |
| Connection Type | Connectionless |
| Reliability | No guaranteed delivery |
| Ordering | No guaranteed ordering |
| Speed | Faster than TCP |
| Use Cases | DNS, streaming, VoIP |

## Procedure:

### Step 1: Create NS3 Simulation Script

A C++ script was created in the NS3 scratch directory with the following components:

- Node Creation: 2 nodes (Client = Node 0, Server = Node 1)

- Point-to-Point Link: 5 Mbps data rate, 2 ms delay

- Internet Stack: Installed on both nodes

- IP Assignment: 10.1.1.1 (Client), 10.1.1.2 (Server)

- UDP Socket Creation: Separate sockets for client and server

- Packet Transmission: 10 packets of 1024 bytes each, 1 second interval

- Callback Functions: ReceivePacket() and SendPacket()

### Step 2: Code Explanation

| Function | Purpose |
| --- | --- |
| ReceivePacket() | Called when server receives a packet; prints packet size and time |
| SendPacket() | Called recursively to send multiple packets; closes socket when done |
| main() | Sets up topology, creates sockets, schedules events, runs simulation |

### Step 3: Run Simulation

The simulation was executed using:

bash

./ns3 run scratch/udp-client-server.cc

## Complete Code: udp-client-server.cc

cpp

/* -*- Mode:C++; c-file-style:"gnu"; indent-tabs-mode:nil; -*- */

#include "ns3/core-module.h"

#include "ns3/network-module.h"

#include "ns3/internet-module.h"

#include "ns3/point-to-point-module.h"

#include "ns3/applications-module.h"

using namespace ns3;

// Function to receive packets on server side

void ReceivePacket(Ptr<Socket> socket)

{

Ptr<Packet> packet;

Address from;

while ((packet = socket->RecvFrom(from)))

{

std::cout << "Server received " << packet->GetSize()

<< " bytes from client at time: "

<< Simulator::Now().GetSeconds() << " seconds\n";

}

}

// Function to send packets from client side

void SendPacket(Ptr<Socket> socket, uint32_t pktSize, uint32_t pktCount, Time pktInterval)

{

if (pktCount > 0)

{

socket->Send(Create<Packet>(pktSize));

std::cout << "Client sent " << pktSize

<< " bytes packet. Packets remaining: " << pktCount - 1 << "\n";

Simulator::Schedule(pktInterval, &SendPacket, socket, pktSize, pktCount - 1, pktInterval);

}

else

{

socket->Close();

std::cout << "Client finished sending all packets.\n";

}

}

int main(int argc, char *argv[])

{

std::cout << "\n========================================\n";

std::cout << "NS3 - UDP CLIENT SERVER COMMUNICATION\n";

std::cout << "========================================\n\n";

Time::SetResolution(Time::NS);

// Step 1: Create 2 nodes (Client and Server)

NodeContainer nodes;

nodes.Create(2);

std::cout << "Step 1: Created 2 nodes (Node 0: Client, Node 1: Server)\n";

// Step 2: Create point-to-point link between nodes

PointToPointHelper p2p;

p2p.SetDeviceAttribute("DataRate", StringValue("5Mbps"));

p2p.SetChannelAttribute("Delay", StringValue("2ms"));

NetDeviceContainer devices = p2p.Install(nodes);

std::cout << "Step 2: Created point-to-point link (5Mbps, 2ms delay)\n";

// Step 3: Install Internet stack on both nodes

InternetStackHelper internet;

internet.Install(nodes);

std::cout << "Step 3: Installed Internet stack on both nodes\n";

// Step 4: Assign IP addresses

Ipv4AddressHelper ipv4;

ipv4.SetBase("10.1.1.0", "255.255.255.0");

Ipv4InterfaceContainer interfaces = ipv4.Assign(devices);

std::cout << "Step 4: Assigned IP addresses - Client: "

<< interfaces.GetAddress(0) << ", Server: "

<< interfaces.GetAddress(1) << "\n";

uint16_t port = 8080;

std::cout << "Step 5: Using port number: " << port << "\n\n";

// Step 6: Create SERVER socket (UDP)

Ptr<Socket> serverSocket = Socket::CreateSocket(nodes.Get(1), UdpSocketFactory::GetTypeId());

InetSocketAddress local = InetSocketAddress(Ipv4Address::GetAny(), port);

serverSocket->Bind(local);

serverSocket->SetRecvCallback(MakeCallback(&ReceivePacket));

std::cout << "Server socket created and listening on port " << port << "\n";

// Step 7: Create CLIENT socket (UDP)

Ptr<Socket> clientSocket = Socket::CreateSocket(nodes.Get(0), UdpSocketFactory::GetTypeId());

InetSocketAddress remote = InetSocketAddress(interfaces.GetAddress(1), port);

clientSocket->Connect(remote);

std::cout << "Client socket created and connected to server at "

<< interfaces.GetAddress(1) << ":" << port << "\n\n";

std::cout << "========================================\n";

std::cout << "Starting Communication...\n";

std::cout << "========================================\n\n";

// Step 8: Schedule packet transmission

// Send 10 packets of 1024 bytes each, 1 second apart

Simulator::Schedule(Seconds(2.0), &SendPacket, clientSocket, 1024, 10, Seconds(1.0));

// Step 9: Stop simulation after 15 seconds

Simulator::Stop(Seconds(15.0));

// Step 10: Run simulation

Simulator::Run();

Simulator::Destroy();

std::cout << "\n========================================\n";

std::cout << "Simulation Completed Successfully!\n";

std::cout << "========================================\n";

return 0;

}

## Output Screenshot:

[Insert ss1_udp_client_server_output.png here]

### Description of Output:

The above screenshot shows the complete terminal output of the UDP client-server simulation. The output can be analyzed in three phases:

Phase 1 - Setup (Steps 1-5):

- 2 nodes were created (Node 0 as Client, Node 1 as Server)

- A point-to-point link was established with 5 Mbps data rate and 2 ms delay

- Internet stack was installed on both nodes

- IP addresses were assigned: Client (10.1.1.1), Server (10.1.1.2)

- UDP port 8080 was selected for communication

Phase 2 - Socket Creation (Steps 6-7):

- Server socket was created and bound to port 8080

- Server began listening for incoming packets

- Client socket was created and connected to server at 10.1.1.2:8080

Phase 3 - Communication (Packet Exchange):

- Client sent 10 packets of 1024 bytes each at 1-second intervals

- Server received each packet and displayed the timestamp

- Packet transmission started at 2.00369 seconds

- Final packet was received at 11.0037 seconds

- Total transmission time: approximately 9 seconds

- Client successfully closed the socket after sending all packets

### Observation:

The simulation shows that UDP provides fast, connectionless communication. Each packet sent by the client was successfully received by the server. The 2 ms link delay contributed to the slight offset in timestamps (2.00369 seconds instead of exactly 2.00 seconds).

## Analysis of Results:

### Packet Transmission Log:

| Packet # | Send Time (sec) | Receive Time (sec) | Delay (ms) |
| --- | --- | --- | --- |
| 1 | 2.00000 | 2.00369 | 3.69 |
| 2 | 3.00000 | 3.00369 | 3.69 |
| 3 | 4.00000 | 4.00369 | 3.69 |
| 4 | 5.00000 | 5.00369 | 3.69 |
| 5 | 6.00000 | 6.00369 | 3.69 |
| 6 | 7.00000 | 7.00369 | 3.69 |
| 7 | 8.00000 | 8.00369 | 3.69 |
| 8 | 9.00000 | 9.00369 | 3.69 |
| 9 | 10.00000 | 10.00370 | 3.70 |
| 10 | 11.00000 | 11.00370 | 3.70 |

### Key Observations:

- Consistent Delay: Each packet experienced approximately 3.69 ms delay, which includes the 2 ms link delay plus processing time.

- No Packet Loss: All 10 packets were successfully transmitted and received, demonstrating reliable communication over this simple point-to-point link.

- Order Preservation: Packets were received in the same order they were sent, though UDP does not guarantee this.

- Real-time Logging: The simulation provided real-time output showing each packet's transmission and reception.

## Comparison: UDP vs TCP

| Parameter | UDP (This Experiment) | TCP |
| --- | --- | --- |
| Connection | Connectionless | Connection-oriented |
| Handshake | No | 3-way handshake |
| Acknowledgment | None (application-level) | Yes |
| Retransmission | No | Yes |
| Flow Control | No | Yes |
| Overhead | Low | High |
| Speed | Faster | Slower |
| Use Case | Real-time, streaming | Reliable file transfer |

## Conclusion:

In this experiment, a UDP-based client-server communication was successfully implemented using NS3 network simulator.

### Key Achievements:

- Topology Setup: Successfully created a point-to-point network with 2 nodes (client and server).

- UDP Socket Programming: Implemented UDP socket creation, binding, connection, and data transmission.

- Callback Functions: Used recursive callback functions (SendPacket) for scheduling multiple packet transmissions.

- Simulation Analysis: Observed packet transmission with consistent 1-second intervals and approximately 3.69 ms per-packet delay.

- Documentation: Captured complete simulation output showing 10 packets of 1024 bytes each being transmitted successfully.

### Learning Outcomes:

- Understood the difference between connectionless (UDP) and connection-oriented (TCP) protocols

- Learned to implement client-server communication in NS3 using raw sockets

- Gained experience with NS3 callback mechanisms and event scheduling

- Analyzed packet timing and delay characteristics in a simulated network

### Applications of UDP:

| Application | Why UDP is Used |
| --- | --- |
| DNS | Fast domain name resolution |
| VoIP | Real-time voice, lost packets acceptable |
| Video Streaming | Speed over reliability |
| Online Gaming | Low latency requirement |
| DHCP | Quick IP address assignment |

## References:

- NS3 Official Documentation - https://www.nsnam.org/doxygen/index.html

- NS3 Spoken Tutorial - https://spoken-tutorial.org

- Computer Networks: A Top-Down Approach - Kurose & Ross

- UDP Protocol - RFC 768

## GitHub Repository:

Repository Link: https://github.com/ShreyasPatil3105/Computer-Networks-Lab